The Manual of Reading and Clearing Code for Honda Motorcycle
[image: image1.jpg]



Reading Codes
1， F1 light checking: switch ignition on，if FI light is on and then off after two seconds it is normal, self testing is also normal. If FI light is still on or twinkling after self testing, there will be fault codes in the ECU.

2,  Steps of reading fault codes: Find the diagnosis connector（DLC connector） as the picture shown[image: image2.png]



Our Honda code reader device has three diagnosis cables，blue, green and brown.

Connect our green cable with the green cable of DLC; connect with our brown cable with the brown cable of DLC. (If there is blue cable but no brown cable in the DLC, please connect our blue cable with the blue cable of DLC, and connect our green cable with the green cable of DLC).Switch our device ON，if FI light will be twinkling regularly，it means that it is reading the codes 
[image: image3.jpg]1 second 0.3 second One fault code

Pulse Width Pulse Width
Light | ’*k4<‘\

i on 4 / ( \

Light | =
Te off 1 N
0.3 second 3 second
time interval time interval





   the fault code’s ID is 23 in the above picture.
Fault codes list:
01. Intake air pressure sensor loop signal short or open circuit
07. Water temperature sensor loop signal short or open circuit
08. Throttle Position Sensor loop signal short or open circuit.
09. Intake air temperature Sensor loop signal short or open circuit.
01&08. Reference supply high potential short or open circuit
01&08&09 Reference supply low potential short or open circuit
12. Injection Nozzle connecting wire short or open circuit, or injection Nozzle coil short or open circuit.
21. O2 sensor connecting wire short or open circuit
29. Idle speed air control so connector wire short or open circuit, or stepping motor coil malfunction.
Clearing codes：
1， Switch ignition off，then connect our green cable with the green cable of DL, connect with our brown cable with the brown cable of DLC.（If there is blue cable but no green cable in the DLC, please connect our blue cable with the blue cable of DLC, and connect our brown cable with the brown cable of DLC）Switch our device on，waiting for 20 seconds。
2， Switch ignition off and switch our device off, FI light will be on about 5 minutes. Switch our device on, then F1light will be offer .If F1 light is twinkling after off, it means that clear fault codes successfully. If not, please repeat the previous steps.
